Section 083613 
Sectional Doors

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional steel doors.

Upward acting sectional aluminum doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Lock cylinders.

- ~~~~~ MASTER NOTE ~~~~~~
This cross-reference is applicable only to doors with electric motor operation.
~~~ END OF MASTER NOTE ~~~~

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Warranty

Manufacturer Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

~~~~~ MASTER NOTE ~~~~~~
  The Overhead Door™ Brand is one of the most recognized and respected brands in the garage door industry. C. G. Johnson invented the upward-lifting garage door in 1921 and the electric garage door opener in 1926. For over 100 years, the Overhead Door™ Brand has stood for product quality and expertise, as well as professional service. From commercial and industrial upward-lifting doors and access systems to residential garage doors and openers, we combine quality, dependable products with reliable service to consistently give home and business owners confidence and peace of mind. To learn more about what our company and our distributors have accomplished over this century of greatness you can visit https://www.overheaddoor100.com/. The Overhead Door™ Brand is a garage door brand of Overhead Door Corporation, a Sanwa Holdings Company. Headquarters are located in Lewisville, TX. With our nationwide network of more than 450 authorized Ribbon Distributors, we are a leading provider of garage door systems, and we continue to lead the way with reliable solutions and unmatched professional installation, service and support that keeps customers coming back.   Contact Information: Overhead Door 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: 1 (800) 929-3667 (DOOR) Email: info@overheaddoor.com   Contact for Design Professional: Technical Support Group Phone: 1-800-929-4692 Email: info@overheaddoor.com   Online CEU Courses   Request Lunch & Learn   https://www.overheaddoor.com/home   Plant Location: Lewistown, Pennsylvania; Williamsport, Pennsylvania; Pensacola, Florida; Grand Island, Nebraska    
~~~ END OF MASTER NOTE ~~~~

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

UPWARD ACTING SECTIONAL STEEL DOORS

Sectional Warehouse Doors:  Overhead Door Corporation; << Model 401;  Model 402;  Model 403;  Model 404;  Model 405; or  Model 406>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 402, 404, and 406 have steel construction with exterior sheet, interior sheet, and core reinforcement.

Models 401, 403, and 405 have steel construction with exterior sheet.
~~~ END OF MASTER NOTE ~~~~

Panels:  << Steel construction with exterior sheet, interior sheet, and core reinforcement; or  Steel construction with exterior sheet>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 401, 402, 403, and 404: 30 feet 2 inches (9.2 m).
Models 405 and 406: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 401, 402, 403, and 404: 22 feet 1 inch (6.7 m).
Models 405 and 406: 16 feet 1 inch (4.9 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

- ~~~~~ MASTER NOTE ~~~~~~
Models 401 and 402: 20 gauge, 0.0355 inch (0.902 mm).
Models 403 and 404: 24 gauge, 0.024 inch (0.610 mm).
Models 405 and 406: Nominal 24 gauge, 0.020 inch (0.508 mm).
~~~ END OF MASTER NOTE ~~~~

Panel Material:  << 20 gauge, 0.0355 inch (0.902 mm);  24 gauge, 0.024 inch (0.610 mm); or  Nominal 24 gauge, 0.020 inch (0.508 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  << 19 inches (483 mm);  21 inches (533 mm); or  24 inches (610 mm)>>.

Insulation:  << Not required; or  Expanded polystyrene>>.

Panel R-value (RSI-value):  << 7.64 (1.35)>>.

Interior Backer:  << Not required;  High impact polystyrene; or  Steel>>.

Exterior Surface Profile:  Ribbed.

Lites:  << Not required;  Vision lites; or  Full-view lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 401 and 402: White Smooth finish only.
Models 403 and 404: White Smooth or Brown Smooth finishes only.
Models 405 and 406: White Smooth, Brown Smooth, White Stucco-Embossed, or Brown Stucco-Embossed.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White smooth;  Brown smooth;  White stucco embossed; or  Brown stucco embossed>>.

Locking Options to Include:  << None;    Slide lock; or    Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or    Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Sectional Steel Doors:  Overhead Door Corporation; << Model 416;  Model 420;  Model 424; or  Model 430>>.

Panels:  Steel construction with exterior sheet.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 416: 32 feet 2 inches (9.8 m).
Model 420: 31 feet 2 inches (9.5 m). 
Model 424: 30 feet 2 inches (9.2 m).
Model 430: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Model 416: 25 feet 1 inch (7.7 m).
Models 420 and 424: 24 feet 1 inch (7.3 m).
Model 426: 24 feet 1 inch (7.3 m).
Model 430: 16 feet 1 inch (4.9 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

- ~~~~~ MASTER NOTE ~~~~~~
Model 416: 16 gauge.
Model 420: 20 gauge.
Model 424: 24 gauge.
Model 430: 26 gauge.
~~~ END OF MASTER NOTE ~~~~

Exterior Steel Thickness:  << 16 gauge, 0.0635 inch (1.613 mm);  20 gauge, 0.0336 inch (0.853 mm);  24 gauge, 0.0216 inch (0.549 mm); or  26 gauge, 0.0187 inch (0.475 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Shiplap.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Model 416:  Flush.
Models 420, 424, 430: Ribbed.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Flush; or  Ribbed>>.

- ~~~~~ MASTER NOTE ~~~~~~
Aluminum sash section with DSB glazing available on Models 416, 420, 424, and 430. 
Insulated DSB vision lites available on Models 216, 420, 424, and 430.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required;  Aluminum sash section lites with double strength billet (DSB) glazing; or  Insulated double strength billet (DSB) vision lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Aluminum sash section: 1/8 inch (3 mm) double strength billet (DSB) glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) tempered DSB glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) acrylic, 1/8 inch (3 mm) and 1/4 inch (6 mm) polycarbonate, and 1/4 inch (6 mm) wired glass. 
Insulated DSB vision lites: 1/2 inch (13 mm) DSB glass, 1/2 inch (13 mm) tempered DSB glass, 1/2 inch (13 mm) acrylic, 1/2 inch (13 mm) polycarbonate.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << 1/8 inch (3 mm) double strength billet glass;  1/2 inch (13 mm) double strength billet glass;  1/8 inch (3 mm) tempered double strength billet glass;  1/4 inch (6 mm) tempered double strength billet glass;  1/2 inch (13 mm) tempered double strength billet glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/2 inch (13 mm) acrylic;  1/8 inch (3 mm) polycarbonate;  1/4 inch (6 mm) polycarbonate;  1/2 inch (13 mm) polycarbonate; or  1/4 inch (6 mm) wired glass>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 416 and 420: White smooth or powder coat smooth finishes.
Models 424 and 430: White smooth, brown smooth, or powder coat smooth finishes.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White smooth;  Brown smooth; or  Powder coat smooth>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

End Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Intermediate Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or    Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at greater than or equal to << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Insulated Sectional Doors:  Overhead Door Corporation; << Model 418;  Model 422;  Model 426; or  Model 432>>.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 418, 422, and 426: 24 feet 2 inches (7.4 m).
Model 432: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 418 and 422: 18 feet 1 inch (5.5 m). 
Model 426: 24 feet 1 inch (7.3 m).
Model 432: 16 feet 1 inch (4.9 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

- ~~~~~ MASTER NOTE ~~~~~~
Model 418: 16 gauge.
Model 422: 20 gauge.
Model 426: 24 gauge.
Model 432: 26 gauge.
~~~ END OF MASTER NOTE ~~~~

Exterior Steel Thickness:  ​<< 16 gauge, 0.0635 inch (1.613 mm);  20 gauge, 0.0236 inch (0.599 mm);  24 gauge, 0.0216 inch (0.549 mm); or  26 gauge, 0.0187 inch (0.475 mm)>>​ galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Shiplap.

Section Height:  Manufacturer's standard based on door height.

Insulation:  Expanded polystyrene.

Panel R-value (RSI-value):  << 7.35 (1.29)>>.

Interior Backer:  << 25 gauge, 0.0184 inch (0.47 mm) painted steel;  Polybonded laminate; or  High impact polystyrene>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 418: Flush.
Models 422, 426, 432: Ribbed.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  ​<< Flush; or  Ribbed>>​.

- ~~~~~ MASTER NOTE ~~~~~~
Aluminum sash section with DSB glazing available on Models 418, 422, 426, and 432.
Insulated DSB vision lites available on Models 418 and 432.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required;  Aluminum sash section lites with double strength billet (DSB) glazing; or  Insulated double strength billet (DSB) vision lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Aluminum sash section: 1/8 inch (3 mm) double strength billet (DSB) glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) tempered DSB glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) acrylic, 1/8 inch (3 mm) and 1/4 inch (6 mm) polycarbonate, and 1/4 inch (6 mm) wired glass.
Insulated DSB vision lites: 1/2 inch (13 mm) DSB glass, 1/2 inch (13 mm) tempered DSB glass, 1/2 inch (13 mm) acrylic, 1/2 inch (13 mm) polycarbonate.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << 1/8 inch (3 mm) double strength billet glass;  1/2 inch (13 mm) double strength billet glass;  1/8 inch (3 mm) tempered double strength billet glass;  1/4 inch (6 mm) tempered double strength billet glass;  1/2 inch (13 mm) tempered double strength billet glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/2 inch (13 mm) acrylic;  1/8 inch (3 mm) polycarbonate;  1/4 inch (6 mm) polycarbonate;  1/2 inch (13 mm) polycarbonate; or  1/4 inch (6 mm) wired glass>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 418 and 422: White smooth or powder coat smooth finish.
Models 426 and 432: White smooth, brown smooth, or powder coat smooth finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White smooth;  Brown smooth; or  Powder coat smooth>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

End Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Intermediate Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>> maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

WindStorm Sectional Doors:  Overhead Door Corporation; << Model 421;  Model 423;  Model 427;  Model 429;  Model 434; or  Model 436>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 423, 429, and 436 have steel construction with exterior sheet, interior sheet, and core reinforcement.
Models 421, 427, and 434 have steel construction with exterior sheet.
~~~ END OF MASTER NOTE ~~~~

Panels:  << Steel construction with exterior sheet, interior sheet, and core reinforcement; or  Steel construction with exterior sheet>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 421, 423, 427, and 429: 24 feet 2 inches (7.3 m).
Models 434 and 436: 20 feet 2 inches (6.2 m). 
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 421, 423, 427, and 429: 26 feet (7.9 m).
Models 434 and 436: 16 feet 1 inch (4.9 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door width for wind testing is 24 feet 2 inches (8.1 m).
For a complete list of wind load pressures by model, please visit manufacturer website.
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand uniform pressure of << _____ psf (_____ kPa)>>.

Impact Resistant:  Yes.

Wind Load:  Not required.

Impact Resistant:  << Required; or  Not required>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

- ~~~~~ MASTER NOTE ~~~~~~
For models 421, 423, 427, and 429, use: 20 gauge, 0.0336 inch (0.853 mm).
For models 434 and 436, use: Nominal 24 gauge, 0.0276 inch (0.701 mm).
~~~ END OF MASTER NOTE ~~~~

Exterior Steel Thickness:  << 20 gauge, 0.0336 inch (0.853 mm); or  Nominal 24 gauge, 0.0276 inch (0.701 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Models 421, 427, and 434: Not insulated.
Models 423, 429, and 436: Polystyrene insulation.
~~~ END OF MASTER NOTE ~~~~

Insulation:  << None; or  Polystyrene>>.

Panel R-value (RSI-value):  << 7.64 (1.35)>>.

Interior Backer:  << Not required;  Galvanized steel; or  High impact polystyrene>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 421 and 423: Ribbed.
Models 427 and 429: Smooth or ribbed.
Models 434 and 436: Smooth, ribbed, or stucco embossed.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Smooth;  Ribbed; or  Stucco embossed>>.

- ~~~~~ MASTER NOTE ~~~~~~
Impact lites required in impact-rated doors.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required;  Full view lites;  Vision lites; or  Impact lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 421 and 423: White finish only.
Models 427, 429, 434, and 436: White or brown finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White; or  Brown>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

End Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Intermediate Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Insulated Steel-Back Sectional Doors:  Overhead Door Corporation; Model 470.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 20 feet (6.1 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height of 14 feet 1 inch (4.3 m). For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door width for wind testing is 18 feet (5.5 m).
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand uniform pressure of << _____ psf (_____ kPa)>>.

Wind Load:  Not required.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

Exterior Steel Thickness:  << 25 gauge, 0.018 inch (0.46 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

Insulation:  Bonded polystyrene.

Panel R-value (RSI-value):  << 9.83 (1.73)>>.

Interior Backer:  Painted galvanized steel, << 30 gauge, 0.011 inch (0.28 mm)>>.

Exterior Surface Profile:  Ribbed.

Lites:  << Not required;  Double thermal acrylic;  Insulated double strength billet (DSB);  Clear short; or  Clear long>>.

- ~~~~~ MASTER NOTE ~~~~~~
Doors with 16.75-inch (4.3 m) sections available only in white and almond.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Almond;  Brown;  Sandstone; or  Desert Tan>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Finish:  Galvanized steel.

End Stiles:  << 20 gauge, 0.032 inch (0.81 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Thermacore Steel Sectional Doors:  Overhead Door Corporation; << Model 591;  Model 592;  Model 593;  Model 594;  Model 596;  Model 598;  Model 599; or  Model 850>>.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 592, 599, and 850: 40 feet 2 inches (12.2 m)
Model 596: 36 feet 2 inches (11.02 m)
Model 591: 35 feet 2 inches (10.7 m)
Model 594: 20 feet 2 inches (6.2 m)
Model 598: 16 feet 2 inches (4.9 m)
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 592 and 599: 32 feet 1 inch (9.8 m) 
Models 591, 596, and 850: 24 feet 1 inch (7.3 m) 
Models 593 and 594: 16 feet 1 inch (4.9 m) 
Model 598: 14 feet 1 inch (4.3 m) 
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Model 591: 0.08 cfm/sq ft (1.46 cu m/hr/sq m)
Model 592, 596, and 599: 0.13 cfm/sq ft (2.37 cu m/hr/sq m)
Models 593 and 594: 0.15 cfm/sq ft (2.74 cu m/hr/sq m)
Model 850: 0.21 cfm/sq ft (3.83 cu m/hr/sq m)
Model 598: 0.46 cfm/sq ft (8.40 cu m/hr/sq m) 
~~~ END OF MASTER NOTE ~~~~

Air Infiltration at << 25 mph (40 km/hr)>>:  << 0.08 cfm/sq ft (1.46 cu m/hr/sq m);  0.13 cfm/sq ft (2.37 cu m/hr/sq m);  0.15 cfm/sq ft (2.74 cu m/hr/sq m);  0.21 cfm/sq ft (3.83 cu m/hr/sq m); or  0.46 cfm/sq ft (8.40 cu m/hr/sq m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 592, 596, and 599: 0.01
Model 850: 0.07
Model 591: 0.13
Models 593 and 594: 0.15
Model 598: 0.20
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << 0.07;  0.10;  0.13;  0.15; or  0.20>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 592, 596, and 599: R-value (RSI-value): 17.40 (3.09).
Model 591: R-value (RSI-value): 14.86 (2.63).
Models 593 and 594: R-value (RSI-value): 12.76 (2.26).
Model 598: R-value (RSI-value): 9.31 (1.64).
Model 850: R-value (RSI-value): 26 (4.58).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 26 (4.58);  17.40 (3.09);  14.86 (2.63);  12.76 (2.26); or  9.31 (1.64)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 592 and 596: Class 26.
Models 591, 593, 594, 598, and 599: Not applicable.
Model 850: Class 22.
~~~ END OF MASTER NOTE ~~~~

STC Rating:  << Class 22;  Class 26; or  Not applicable>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:

- ~~~~~ MASTER NOTE ~~~~~~
Models 594 and 598: 0.012 inch (0.30 mm).
Models 591, 592, 593, 599, and 850: 0.015 inch (0.38 mm).
Model 596: 0.036 inch (0.91 mm).
~~~ END OF MASTER NOTE ~~~~

Exterior Steel Thickness:  << 0.012 inch (0.30 mm);  0.015 inch (0.38 mm); or  0.033 inch (0.84 mm)>> galvanized steel.

- ~~~~~ MASTER NOTE ~~~~~~
Models 592, 596, and 599: 2 inches (51 mm).
Model 591: 1-5/8 inches (41 mm).
Models 593 and 594: 1-3/8 inches (35 mm). 
Model 598: 1 inch (25 mm).
Model 850: 3 inches (76 mm).
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 3 inches (76 mm);  2 inches (51 mm);  1-5/8 inches (41 mm);  1-3/8 inches (35 mm); or  1 inch (25 mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Tongue and groove joint profile for Model 850.
Shiplap joint profile for Models 592, 596, 599, 593, and 598.
~~~ END OF MASTER NOTE ~~~~

Joint Profile:  << Tongue and groove; or  Shiplap>>.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Models 596 and 599: Flush.
Model 850: Microgroove.
Model 594: Raised panel. 
Models 591, 592, 593, and 598: Ribbed.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Flush;  Microgroove;  Raised panel; or  Ribbed>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 850: Large lites or insulated lites.
Models 596 and 599: Double thermal acrylic, aluminum sash section with DSB glazing, insulated DSB, or clear long lites.
Models 591: Double thermal acrylic, aluminum sash section with DSB glazing, or insulated DSB.
Models 592 and 593: Double thermal acrylic, aluminum sash section with DSB glazing, insulated DSB, or clear long lites. Clear long lites not available on doors over 20 feet long.
Model 598: Double thermal acrylic, insulated DSB, or clear long lites. Clear long lites not available on doors over 20 feet long. 
Model 594: Insulated DSB or aluminum sash section with DSB glazing.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required;  Double thermal acrylic;  Aluminum sash section with double strength billet (DSB) glazing;  Insulated double strength billet (DSB);  Clear long lites; or  Large lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 591 standard finish colors: White, Tan, Gray, Industrial Brown, Black. Optional Finish colors: Trinar White, Trinar Brown, Trinar Beige.
Model 592 standard finish colors: White, Tan, Gray, Industrial Brown, Black. Optional Finish colors: Trinar White, Trinar Brown, Trinar Beige.
Model 593 standard finish colors: White, Tan, Gray, Industrial Brown. Optional Finish colors: Trinar White, Trinar Brown, Trinar Beige.
Model 594 standard finish colors: White, Almond, Sandstone, Forest Green, Brown, Terra Bronze, Desert Tan. Optional Finish colors: Trinar White, Trinar Brown, Trinar Beige.
Model 596 standard finish colors: White, Tan, Gray, Industrial Brown.
Model 598 standard finish colors: White.
Model 599 standard finish colors: White.
Model 850 standard finish colors: White, Tan,  Brown.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Tan;  Gray;  Almond;  Industrial Brown;  Black;  Sandstone;  Forest Green;  Chestnut Brown;  Terra Bronze;  Desert Tan;  Trinar White;  Trinar Brown; or  Trinar Beige>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

- ~~~~~ MASTER NOTE ~~~~~~
Models 591, 592, 596, and 599: 0.0555 inch (1.410 mm).
Models 593, 594, and 598: 0.0356 inch (0.904 mm).
Model 850: single 16 gauge, 0.0635 inch (1.613 mm); double 16 gauge, 0.0635 inch (1.613 mm); or 14 gauge, 0.0785 inch (1.99 mm).
~~~ END OF MASTER NOTE ~~~~

End Stiles:  << 14 gauge, 0.0785 inch (1.99 mm);  single 16 gauge, 0.0635 inch (1.613 mm);  double 16 gauge, 0.0635 inch (1.613 mm);  0.055 inch (1.410 mm); or  0.0356 inch (0.904 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Thermacore Windstorm Steel Sectional Doors:  Overhead Door Corporation; << Model 515; or  Model 525>>.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 525: 22 feet 2 inches (6.8 m).
Model 515: 20 feet (6.1 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height: 20 feet 1 inch (6.1 m) for models 515 and 525.
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 0.23 cfm/sq ft (4.20 cu m/hr/sq m).
Models 525: 0.07 cfm/sq ft (1.28 cu m/hr/sq m).
~~~ END OF MASTER NOTE ~~~~

Air Infiltration at << 15 mph (24 km/hr)>>:  << 0.23 cfm/sq ft (4.20 cu m/hour/sq m); or  0.07 cfm/sq ft (1.28 cu m/hour/sq m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 0.15.
Model 525: 0.12.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << 0.12; or  0.15>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: R-value (RSI-value): 12.12 (2.13).
Model 525: R-value (RSI-value): 16.22 (2.85).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 12.12 (2.13); or  16.22 (2.85)>>.

STC Rating:  Class 20.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Ten years against delamination and cracking, splitting, or deterioration due to rust-through.

Panel Material:

Exterior Steel Thickness:  << 0.015 inch (0.38 mm)>> galvanized steel.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 1-3/8 inches (35 mm).
Model 525: 1-7/8 inch (48 mm).
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 1-3/8 inches (35 mm); or  1-7/8 inches (48 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Models 515 and 525 are available with embossed wood grain or stucco embossed pinstripe finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Embossed wood grain; or  Stucco embossed pinstripe>>.

- ~~~~~ MASTER NOTE ~~~~~~
V10 panel available on Model 515 only.
~~~ END OF MASTER NOTE ~~~~

Panel Profile:  << Standard;  V5 panel;  Long panel;  Flush panel; or  V10 panel>>.

- ~~~~~ MASTER NOTE ~~~~~~
Various lite styles available. For a complete list of options, visit manufacturer website.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options available with Standard, Long, and Flush panels: White, Almond, Taupe, Brown, Terra Bronze, or Black (model 515 only).
Finish options available with V5 panel, model 515: Golden Oak, Walnut, or Mission Oak.
Finish options available with Microgroove panel:  White textured finish, Almond textured finish, Taupe textured finish, Brown textured finish, or Black textured finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Almond;  Taupe;  Brown;  Terra Bronze;  Black;  Golden Oak;  Walnut;  Mission Oak;  White textured finish;  Almond textured finish;  Taupe textured finish;  Brown textured finish; or  Black textured finish>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

End Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Intermediate Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

UPWARD ACTING Sectional Aluminum Doors

Aluminum Sectional Doors:  Overhead Door Corporation; << Model 511;  Model 521; or  Model 522>>.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 511: 16 feet 2 inches (6.2 m).
Model 521: 26 feet 2 inches (8 m).
Model 522: 18 feet 2 inches (5.5 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights: 
Model 511: 16 feet 1 inch (4.9 m).
Model 521: 20 feet 1 inch (6.1 m).
Model 522: 14 feet 1 inch (4.3 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
U-factor values:
Model 521 with double strength billet (DSB) clear, tempered, or obscure glazing: 0.30.
Model 521 with DSB Low-E coated glazing: 0.28.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << Not required;  0.30; or  0.28>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 521:
With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

- ~~~~~ MASTER NOTE ~~~~~~
Models 511 and 521: 1-3/4 inches (45 mm).
Model 522: 1-3/8 inches (35 mm). 
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 1-3/8 inches (35 mm); or  1-3/4 inches (45 mm)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 511 and 521 have shiplap joint profile.
Model 522 has tongue and groove joint profile.
~~~ END OF MASTER NOTE ~~~~

Joint Profile:  << Tongue and groove; or  Shiplap>>.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Glazing options available per model:
Models 511 and 521: 1/8 inch (3 mm) DSB glass, 1/8 inch (3 mm) and 1/4 inch (6 mm) tempered DSB glass, 1/8 inch (3mm) and 1/4 inch (6 mm) acrylic, 1/8 inch (3 mm) and 1/4 inch (6 mm) clear polycarbonate, 1/4 inch (6 mm) wired glass.
Insulated DSB vision lites: 1/2 inch (13 mm) DSB glass, 1/2 inch (13 mm) tempered DSB glass, 1/2 inch (13 mm) acrylic, 1/2 inch (13 mm) polycarbonate.

Model 522: 1/4 inch (6 mm) opaque white, opaque black, mirrored gray, mirrored bronze, translucent black. DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic;  1/2 inch (13 mm) polycarbonate;  1/4 inch (6 mm) opaque white;  1/4 inch (6 mm) opaque black;  1/4 inch (6 mm) mirrored gray;  1/4 inch (6 mm) mirrored bronze; or  1/4 inch (6 mm) translucent black>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options per model:
Models 511 and 521: Clear anodized, light bronze anodized, medium bronze anodized, dark bronze anodized, black anodized, powder coat.
Model 522:  Dark bronze anodized, black anodized, powder coated white, powder coated black, powder coated bronze.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

Aluminum Sectional Doors:  Overhead Door Corporation; Model 521S.

Panels:  Aluminum construction and reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height: 18 feet 1 inch (5.5 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at speed of << 15.7 inches (399 mm)>> per second.

Closing Speed:  Door to operate at speed of << 10.4 inches (264 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
U-factor values:
With DSB clear, tempered, or obscure glazing: 0.30.
With DSB Low-E coated glazing: 0.28. DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << 0.30; or  0.28>>.

- ~~~~~ MASTER NOTE ~~~~~~
R-value (RSI-value) for Model 521S:
With double strength billet (DSB) clear, tempered, or obscure glazing IGUs: R-value (RSI-value): 2.87 (0.51).
With clear polycarbonate IGUs: R-value (RSI-value): 2.93 (0.52).
With solar bronze IGUs: R-value (RSI-value): 3.17 (0.56).
With Low-E coating IGUs: R-value (RSI-value): 3.43 (0.60).
With SolarBan 70XL argon-filled IGUs: R-value (RSI-value): 4.09 (0.72).
With 1/4-inch thick multi-wall polycarbonate: R-value (RSI-value): 2.75 (0.48).
With 3/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.21 (0.57).
With 5/8-inch thick multi-wall polycarbonate: R-value (RSI-value): 3.48 (0.61).
With 3/8-inch extruded polystyrene-filled solid panels: R-value (RSI-value): 2.60 (0.46).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 2.60 (0.46);  2.75 (0.48);  2.87 (0.51);  2.93 (0.52);  3.17 (0.56);  3.21 (0.57);  3.43 (0.60);  3.48 (0.61); or  4.09 (0.72)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum 300,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

Panel Material:  Extruded 6063-T6 aluminum.

Panel Thickness:  << 1-3/4 inches (45 mm)>>.

Joint Profile:  Shiplap.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
DSB: Double strength billet.
~~~ END OF MASTER NOTE ~~~~

Glazing:  << Not required;  1/8 inch (3 mm) DSB glass;  1/8 inch (3 mm) tempered DSB glass;  1/4 inch (6 mm) tempered DSB glass;  1/8 inch (3 mm) acrylic;  1/4 inch (6 mm) acrylic;  1/8 inch (3 mm) clear polycarbonate;  1/4 inch (6 mm) clear polycarbonate;  1/4 inch (6 mm) wired glass;  1/2 inch (13 mm) DSB glass;  1/2 inch (13 mm) tempered DSB glass;  1/2 inch (13 mm) acrylic; or  1/2 inch (13 mm) polycarbonate>>.

Exterior Finish:  << Clear anodized;  Light bronze anodized;  Medium bronze anodized;  Dark bronze anodized;  Black anodized; or  Powder coat; color: _____>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Manual Operation:  Chain Hoist.

Motor Operation:  Provide UL-listed direct drive, integrated gear motor and brake electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second. Provide operator and drive assembly with all wiring harnesses needed from manufacturer.

Timer to Close:  Automatic closing controlled by adjustable hold-open time delay.

Operation Supply Voltage:  << 208V 1 phase 60 Hz, 190 to 207V range;  208V 3 phase 60 Hz, 190 to 207V range;  230V 1 phase 60 Hz, 208 to 245V range;  230V 3 phase 60 Hz, 208 to 245V range;  460V 3 phase 60 Hz, 456 to 495V range; or  575V 3 phase 60 Hz>>.

Control Panel:  << Medium BTR Control; or  Full BTR Control>> Electronic controller with microprocessor self-diagnostics. Digital readout indicates door action, alarm conditions and fault conditions.

Signaling Device:  << Horn and strobe combination;  Traffic warning light; or  LED Lights>>.

Actuation Device:  Provide << push button;  key switch;  vehicle detection;  loop detector;  motion detector;  treadle switch;  radio control;  door timer; and  photocell operation>>.

Motor Mounting:  Shaft mount.

Obstruction Safety Detection:

Primary, Monitored:

Safety Edge:  << Wireless electric sensing edge; or  2-wire sensing edge>> complying with <<UL 325>>, 2010 edition.

Secondary, Non-Monitored:

Photoelectric Sensors:  << Reflective;  Dual; or  _______>>.

PART 3 EXECUTION

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  ___________>>.

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

END OF SECTION