Section 083323
upward Coiling Doors
Model 600 Coil-awaytm

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward coiling service doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Cylinder cores and keys.

Section << 260583 - Wiring Connections>>:  Power to disconnect.

Reference Standards

ASTM A653/A653M - Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process; 2023.

ASTM E330/E330M - Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference; 2014 (Reapproved 2021).

FBC TAS 201 - Impact Test Procedures; Testing Application Standard; 1994.

FBC TAS 202 - Criteria for Testing Impact and Non-Impact Resistant Building Envelope Components Using Uniform Static Air Pressure; Testing Application Standard; 1994.

UL (DIR) - Online Certifications Directory; Current Edition.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Quality Assurance

Manufacturer Qualifications:  Company specializing in performing work of this section with minimum of << 5 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience in fabrication and installation of security closures.

Installer Qualifications:  Company specializing in performing work of this section with minimum of << 3 years; or  _____ years>> of<<  documented;    _____; or  None - N/A>> experience and approved by manufacturer.

Products Requiring Electrical Connection:  Listed and classified by << UL (DIR);  TUV;  testing firm acceptable to authorities having jurisdiction; or  _____>> as suitable for purpose specified.

Delivery, Storage, and Handling

Store products in manufacturer's unopened packaging until ready for installation.

Protect materials from exposure to moisture.

Store materials in dry, warm, ventilated, weathertight location.

Warranty

Manufacturer Door-Only Warranty:  Provide manufacturer warranty for door assembly for duration indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

Manufacturer Door and Operator Warranty:  Provide manufacturer's limited warranty for door and operator system free from material and workmanship defects for duration and cycles indicated under individual doors; counterbalance spring and finish not covered by warranty.

PART 2 PRODUCTS

Manufacturer

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

upward Coiling Service Doors

Overhead DoorTM Brand; Model 600 Coil-Away.

Slats:  Interlocking, roll-formed, galvanized steel slats.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 16 feet (4.9 m). For larger doors, contact manufacturer.

- ~~~ END OF MASTER NOTE ~~~~

Width:   ____ feet (____ mm).

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum width of 16 feet (4.9 m). For larger doors, contact manufacturer.

- ~~~ END OF MASTER NOTE ~~~~

Height:   ____ feet (____ mm).

Mounting:  << Surface-mounted on side indicated on drawings; or  Between jamb>>.

Opening Speed:  Door to operate at variable speed up to << 8 inches (203 mm)>> per second.

Closing Speed:  Door to operate at variable speed up to << 12 inches (305 mm)>> per second.

Operation Cycles:  Capable of operating for minimum 10,000 cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Two years.

Door and Operator Warranty:  Three years or 20,000 cycles, whichever comes first.

- ~~~~~ MASTER NOTE ~~~~~~
Maximum door widths for available wind load ratings:
20 psf: 16 feet (4.88 m)
26 psf: 16 feet (4.88 m)
31 psf: 14 feet (4.27 m)
36 psf: 12 feet (3.66 m)
41 psf: 10 feet (3.05 m)
~~~ END OF MASTER NOTE ~~~~

Wind Load:  Design door assembly to withstand ultimate static pressure load of << 20 psf (0.9 kPa);  26 psf (1.2 kPa);  31 psf (1.4 kPa);  36 psf (1.7 kPa); or  41 psf (1.9 kPa)>> in accordance with <<ASTM E330/E330M>>.

Wind Load:  Not required.

Curtain Material:  Galvanized steel, << 26 gauge, 0.217 inch (0.55 mm); or  _____ gauge, _____ inch (_____ mm)>> for widths up to << 12 feet 4 inches (3.75 m)>>, << 24 gauge, 0.278 inch (0.63 mm); or  _____ gauge, _____ inch (_____ mm)>> for widths up to << 16 feet (4.9 m)>>. Staking lock system locks end of each slat from lateral movement.

Slat Profile:  Flat crown profile, type CAW.

Galvanized Steel Finish:

Standard polyester base coat.

Color:  << White interior and exterior;  White interior and brown exterior; or  Brown interior and white exterior>>.

PowderGuard Premium powder coat.

- ~~~~~ MASTER NOTE ~~~~~~
PowderGuard Premium polyester powder coat available in 197 colors; custom color match options available.
~~~ END OF MASTER NOTE ~~~~

Color: Color:  << _______>>.

Bottom Bar:  6063-T6 aluminum.

Profile:  << Extruded T; or  Extruded tube>>.

Finish:  << Mill finish;  Clear anodized;  Black anodized;  Bronze anodized; or  PowderGuard Premium powder coat; _____ color>>.

Locking Options to Include:  << None;  Slide lock;  Cylinder; or  Best cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Lock Access:  << Coil side; or  Opposite coil side>>.

Weatherstripping and Seals:

Bottom Seal Astragal:  << Required; or  Not required>>.

Lintel Brush Seal:  << Required; or  Not required>>.

Side Guides, Channels:  Constructed of steel with members fully bolted together.

Finish:  << Galvanized; or  Powder coated; _____ color>>.

Brackets:  Steel to support counterbalance and curtain.

Finish:  << Galvanized; or  Powder coated; _____ color>>.

Counterbalance:  Helical torsion spring type, housed in steel tube or pipe barrel and supporting curtain with deflection limited to << 0.03 inch per foot (1:400 mm/sec)>> of span. Adjustable spring tension required.

Hood:  << Not required; or  24-gauge, 0.0278 inch galvanized steel  (0.63 mm galvanized steel)>>.

Finish:  << Match curtain;  Standard polyester base coat; white color;  Standard polyester base coat; brown color; or  PowderGuard Premium powder coat; _____ color>>.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up available up to 100 sq ft (9.29 sq m) or 10 by 10 feet (3.05 by 3.05 m). Chain hoist over 100 sq ft (9.29 sq m) or 10 by 10 feet (3.05 by 3.05 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  << Not required; or  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum 8 inches (203 mm), maximum 1 foot (305 mm) per second>>.

Manual Override:  << Hoist;  Hoist and jackshaft release; or  Jackshaft release>>.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1-phase 60Hz;  208/230/460V 3-phase 60Hz;  575V 3-phase 60Hz;  220V 1-phase 50Hz;  220V 3-phase 50Hz; or  400V 3-phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall-mounted; or  Front of hood>>, << right-hand side; or  left-hand side>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Constant contact button only.

Obstruction Safety Detection:

Primary, Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  NEMA 4X photo-eyes>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge;  Wireless reversing safety edge; or  ______>>.

Secondary, Non-Monitored:

Photo Eyes:  << Reflective photo-eyes;  Thru-beam photo-eyes; or  _____>>.

Light Curtain:  << 3 feet (0.9 m);  6 feet (1.8 m); or  _____ feet (_____ m)>>.

Safety Edge:  << 2-wire reversing safety edge; or  ______>>.

PART 3 EXECUTION

Examination

Verify << existing conditions; and  _______>> meet manufacturer's requirements before starting work.

Verify opening sizes, tolerances and conditions are acceptable.

Installation

Install in accordance with manufacturer's written instructions.

Use anchorage devices to securely fasten assembly to wall construction and building structure without distortion or stress.

Securely and rigidly brace components suspended from structure.<<  Secure guides to structural members only.;    ______ .; or  None - N/A>>

Fit and align assembly including hardware; level and plumb, to provide smooth operation.

Coordinate installation of electrical service; see Section 260583.

Install enclosure and perimeter trim.

TOLERANCES

Maintain dimensional tolerances and alignment with adjacent work.

Maximum Variation from Plumb:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.6 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch per 10 feet (3.2 mm per 3 m); or  ____ inch per 10 feet (____ mm per 3 m)>>.

adjusting

Adjust operating assemblies for smooth and noiseless operation.

CLeaning

See Section 017000 - Execution and Closeout Requirements for additional requirements.

Clean installed components.

Remove labels and visible markings.

PROTECTION

Protect finishes until completion of project.

Touch up damaged finishes after Substantial Completion.

END OF SECTION