Section 083613 
Sectional WAREHOUSE Doors
Models 401-406

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional steel doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Lock cylinders.

- ~~~~~ MASTER NOTE ~~~~~~
This cross-reference is applicable only to doors with electric motor operation.
~~~ END OF MASTER NOTE ~~~~

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Warranty

Manufacturer Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

~~~~~ MASTER NOTE ~~~~~~
  The Overhead Door™ Brand is one of the most recognized and respected brands in the garage door industry. C. G. Johnson invented the upward-lifting garage door in 1921 and the electric garage door opener in 1926. For over 100 years, the Overhead Door™ Brand has stood for product quality and expertise, as well as professional service. From commercial and industrial upward-lifting doors and access systems to residential garage doors and openers, we combine quality, dependable products with reliable service to consistently give home and business owners confidence and peace of mind. To learn more about what our company and our distributors have accomplished over this century of greatness you can visit https://www.overheaddoor100.com/. The Overhead Door™ Brand is a garage door brand of Overhead Door Corporation, a Sanwa Holdings Company. Headquarters are located in Lewisville, TX. With our nationwide network of more than 450 authorized Ribbon Distributors, we are a leading provider of garage door systems, and we continue to lead the way with reliable solutions and unmatched professional installation, service and support that keeps customers coming back.   Contact Information: Overhead Door 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: 1 (800) 929-3667 (DOOR) Email: info@overheaddoor.com   Contact for Design Professional: Technical Support Group Phone: 1-800-929-4692 Email: info@overheaddoor.com   Online CEU Courses   Request Lunch & Learn   https://www.overheaddoor.com/home   Plant Location: Lewistown, Pennsylvania; Williamsport, Pennsylvania; Pensacola, Florida; Grand Island, Nebraska    
~~~ END OF MASTER NOTE ~~~~

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

UPWARD ACTING SECTIONAL STEEL DOORS

Sectional Warehouse Doors:  Overhead Door Corporation; << Model 401;  Model 402;  Model 403;  Model 404;  Model 405; or  Model 406>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 402, 404, and 406 have steel construction with exterior sheet, interior sheet, and core reinforcement.

Models 401, 403, and 405 have steel construction with exterior sheet.
~~~ END OF MASTER NOTE ~~~~

Panels:  << Steel construction with exterior sheet, interior sheet, and core reinforcement; or  Steel construction with exterior sheet>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Models 401, 402, 403, and 404: 30 feet 2 inches (9.2 m).
Models 405 and 406: 20 feet 2 inches (6.2 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum heights:
Models 401, 402, 403, and 404: 22 feet 1 inch (6.7 m).
Models 405 and 406: 16 feet 1 inch (4.9 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  One year.

- ~~~~~ MASTER NOTE ~~~~~~
Models 401 and 402: 20 gauge, 0.0355 inch (0.902 mm).
Models 403 and 404: 24 gauge, 0.024 inch (0.610 mm).
Models 405 and 406: Nominal 24 gauge, 0.020 inch (0.508 mm).
~~~ END OF MASTER NOTE ~~~~

Panel Material:  << 20 gauge, 0.0355 inch (0.902 mm);  24 gauge, 0.024 inch (0.610 mm); or  Nominal 24 gauge, 0.020 inch (0.508 mm)>> galvanized steel.

Panel Thickness:  << 2 inches (51 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  << 19 inches (483 mm);  21 inches (533 mm); or  24 inches (610 mm)>>.

Insulation:  << Not required; or  Expanded polystyrene>>.

Panel R-value (RSI-value):  << 7.64 (1.35)>>.

Interior Backer:  << Not required;  High impact polystyrene; or  Steel>>.

Exterior Surface Profile:  Ribbed.

Lites:  << Not required;  Vision lites; or  Full-view lites>>.

- ~~~~~ MASTER NOTE ~~~~~~
Models 401 and 402: White Smooth finish only.
Models 403 and 404: White Smooth or Brown Smooth finishes only.
Models 405 and 406: White Smooth, Brown Smooth, White Stucco-Embossed, or Brown Stucco-Embossed.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White smooth;  Brown smooth;  White stucco embossed; or  Brown stucco embossed>>.

Locking Options to Include:  << None;    Slide lock; or    Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or    Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

PART 3 EXECUTION

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  ___________>>.

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

END OF SECTION