Section 083613 
Thermacore Windstorm Steel Sectional Doors
Models 515, 525

NOTE: Enable Microsoft Word option to display hidden text to view notes to specifier.

PART 1 GENERAL

Section Includes

Upward acting sectional steel doors.

Related Requirements

Section << 079200 - Joint Sealants>>:  Sealing joints between frames and adjacent construction.

Section << 087100 - Door Hardware>>:  Lock cylinders.

- ~~~~~ MASTER NOTE ~~~~~~
This cross-reference is applicable only to doors with electric motor operation.
~~~ END OF MASTER NOTE ~~~~

Section << 260533.13 - Conduit for Electrical Systems>>:  Conduit from electric circuit to operator and from operator to control station, and empty conduit from control units to door operator.

Section << 260583 - Wiring Connections>>.

Reference Standards

ITS (DIR) - Directory of Listed Products; Current Edition.

UL (DIR) - Online Certifications Directory; Current Edition.

UL 325 - Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems; Current Edition, Including All Revisions.

~~~~~ MASTER NOTE ~~~~~~
Do not request submittals if drawings sufficiently describe the products of this section or if proprietary specifying techniques are used. The review of submittals increases the possibility of unintended variations from contract documents, thereby increasing the design professional's liability.

- ~~~ END OF MASTER NOTE ~~~~

Submittals

Product Data:  Submit manufacturer's standard literature showing materials and details of construction and finish.<<  Include data on electrical operation.; or  None - N/A>>

Shop Drawings:  Indicate rough and actual opening dimensions, anchorage methods, hardware locations, and installation details.

Manufacturer's Installation Instructions:  Indicate installation sequence and installation, adjustment, and alignment procedures.

Installer's qualification statement.

Operation and Maintenance Data:  Indicate modes of operation, lubrication requirements and frequency, and periodic adjustments required.

Specimen warranty.

Warranty

Manufacturer Warranty:  Provide manufacturer warranty for counterbalance shaft assembly for years indicated under individual doors. Complete forms in <<Owner>>'s name and register with manufacturer.

PART 2 PRODUCTS

Manufacturer

~~~~~ MASTER NOTE ~~~~~~
  The Overhead Door™ Brand is one of the most recognized and respected brands in the garage door industry. C. G. Johnson invented the upward-lifting garage door in 1921 and the electric garage door opener in 1926. For over 100 years, the Overhead Door™ Brand has stood for product quality and expertise, as well as professional service. From commercial and industrial upward-lifting doors and access systems to residential garage doors and openers, we combine quality, dependable products with reliable service to consistently give home and business owners confidence and peace of mind. To learn more about what our company and our distributors have accomplished over this century of greatness you can visit https://www.overheaddoor100.com/. The Overhead Door™ Brand is a garage door brand of Overhead Door Corporation, a Sanwa Holdings Company. Headquarters are located in Lewisville, TX. With our nationwide network of more than 450 authorized Ribbon Distributors, we are a leading provider of garage door systems, and we continue to lead the way with reliable solutions and unmatched professional installation, service and support that keeps customers coming back.   Contact Information: Overhead Door 2501 S State Hwy 121 Bus, Suite 200 Lewisville, Texas 75067 United States   Phone: 1 (800) 929-3667 (DOOR) Email: info@overheaddoor.com   Contact for Design Professional: Technical Support Group Phone: 1-800-929-4692 Email: info@overheaddoor.com   Online CEU Courses   Request Lunch & Learn   https://www.overheaddoor.com/home   Plant Location: Lewistown, Pennsylvania; Williamsport, Pennsylvania; Pensacola, Florida; Grand Island, Nebraska    
~~~ END OF MASTER NOTE ~~~~

Overhead Door Corporation; www.overheaddoor.com; 1 (800) 929-3667.

Substitutions: Not permitted.

UPWARD ACTING SECTIONAL STEEL DOORS

Thermacore Windstorm Steel Sectional Doors:  Overhead Door Corporation; << Model 515; or  Model 525>>.

Panels:  Steel construction with exterior sheet, interior sheet, and core reinforcement.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum widths:
Model 525: 22 feet 2 inches (6.8 m).
Model 515: 20 feet (6.1 m).
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Width:  << _____ feet _____ inches (_____ m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Standard maximum height: 20 feet 1 inch (6.1 m) for models 515 and 525.
For larger doors, contact manufacturer.
~~~ END OF MASTER NOTE ~~~~

Height:  << _____ feet _____ inch (_____ m)>>.

Mounting:  Surface mounted on interior of building.

Opening Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

Closing Speed:  Door to operate at variable speed of << 6 to 12 inches (152 to 305 mm)>> per second.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 0.23 cfm/sq ft (4.20 cu m/hr/sq m).
Models 525: 0.07 cfm/sq ft (1.28 cu m/hr/sq m).
~~~ END OF MASTER NOTE ~~~~

Air Infiltration at << 15 mph (24 km/hr)>>:  << 0.23 cfm/sq ft (4.20 cu m/hour/sq m); or  0.07 cfm/sq ft (1.28 cu m/hour/sq m)>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 0.15.
Model 525: 0.12.
~~~ END OF MASTER NOTE ~~~~

U-Factor:  << 0.12; or  0.15>>.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: R-value (RSI-value): 12.12 (2.13).
Model 525: R-value (RSI-value): 16.22 (2.85).
~~~ END OF MASTER NOTE ~~~~

R-value (RSI-value):  << 12.12 (2.13); or  16.22 (2.85)>>.

STC Rating:  Class 20.

- ~~~~~ MASTER NOTE ~~~~~~
Select operation cycles to coordinate with selected springs.
~~~ END OF MASTER NOTE ~~~~

Operation Cycles:  Capable of operating for minimum << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles. One operation cycle is complete when door is opened from closed position to fully open position and returned to closed position.

Available Warranty:

Door Only:  Ten years against delamination and cracking, splitting, or deterioration due to rust-through.

Panel Material:

Exterior Steel Thickness:  << 0.015 inch (0.38 mm)>> galvanized steel.

- ~~~~~ MASTER NOTE ~~~~~~
Model 515: 1-3/8 inches (35 mm).
Model 525: 1-7/8 inch (48 mm).
~~~ END OF MASTER NOTE ~~~~

Panel Thickness:  << 1-3/8 inches (35 mm); or  1-7/8 inches (48 mm)>>.

Joint Profile:  Tongue and groove.

Section Height:  Manufacturer's standard based on door height.

- ~~~~~ MASTER NOTE ~~~~~~
Models 515 and 525 are available with embossed wood grain or stucco embossed pinstripe finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Surface Profile:  << Embossed wood grain; or  Stucco embossed pinstripe>>.

- ~~~~~ MASTER NOTE ~~~~~~
V10 panel available on Model 515 only.
~~~ END OF MASTER NOTE ~~~~

Panel Profile:  << Standard;  V5 panel;  Long panel;  Flush panel; or  V10 panel>>.

- ~~~~~ MASTER NOTE ~~~~~~
Various lite styles available. For a complete list of options, visit manufacturer website.
~~~ END OF MASTER NOTE ~~~~

Lites:  << Not required; or  _______>>.

- ~~~~~ MASTER NOTE ~~~~~~
Finish options available with Standard, Long, and Flush panels: White, Almond, Taupe, Brown, Terra Bronze, or Black (model 515 only).
Finish options available with V5 panel, model 515: Golden Oak, Walnut, or Mission Oak.
Finish options available with Microgroove panel:  White textured finish, Almond textured finish, Taupe textured finish, Brown textured finish, or Black textured finish.
~~~ END OF MASTER NOTE ~~~~

Exterior Finish:  << White;  Almond;  Taupe;  Brown;  Terra Bronze;  Black;  Golden Oak;  Walnut;  Mission Oak;  White textured finish;  Almond textured finish;  Taupe textured finish;  Brown textured finish; or  Black textured finish>>.

Locking Options to Include:  << None;  Slide lock; or  Cylinder>>.

Lock Locations:  << Both jambs;  Left only; or  Right only>>.

Weatherstripping and Seals:  << Required; or  Not required>>.

Tracks:  Constructed of steel with members fully bolted together.

Size:  << 2 inches (51 mm); or  3 inches (76 mm)>>.

Finish:  Mill finish.

End Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Intermediate Stiles:  << 16 gauge, 0.0635 inch (1.613 mm)>> galvanized steel.

Brackets:  Steel to support counterbalance and curtain.

Finish:  Hot-dip galvanized.

Springs:  Rated for << 10,000;  25,000;  50,000;  75,000; or  100,000>> cycles.

- ~~~~~ MASTER NOTE ~~~~~~
Manual push-up to 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m). Chain hoist over 84 sq ft (7.8 sq m) or 12 by 7 feet (3.7 by 2.1 m).
~~~ END OF MASTER NOTE ~~~~

Manual Operation:  << Push-up; or  Chain hoist>>.

Motor Operation:  Provide UL-listed electric operator; size as recommended by manufacturer to move door in either direction at minimum << 6 inches (152 mm)>>, maximum << 12 inches (305 mm)>> per second.

Timer to Close:  << Not required; or  Automatic closing controlled by adjustable hold-open time delay>>.

Operation Supply Voltage:  << 115/208/230V 1 phase 60Hz;    208/230/460V 3 phase 60Hz;    575V 3 phase 60Hz;    220V 1 phase 50Hz;    220V 3 phase 50Hz; or    400V 3 phase 50Hz>>.

Signaling Device:  << Horn and strobe combination; or  Traffic warning light>>.

Actuation Device:  Provide << push button;  key switch;  pull cord;  loop detector;  motion detector;  treadle switch; and  radio control>>.

Motor Mounting:  << Wall mounted; or  Trolley>>, << right-hand side;  left-hand side; or  center>>.

Motor Enclosure:  << Not required;  NEMA 4-12 watertight and oiltight;  NEMA 4X corrosion-resistant;  Totally enclosed, fan-cooled motor; or  Totally enclosed, non-ventilated>>.

Obstruction Safety Detection:  Infrared sensor mounted in-plane to door curtain at << 6 inches (152 mm)>> from floor.

Reversing Safety Edge:  << Not required;  Door provided with wireless monitored electric safety edge; or  Door provided with wired monitored electric safety edge>>.

PART 3 EXECUTION

Examination

Verify wall openings are ready to receive work and opening dimensions and tolerances are within specified limits.

Verify electric power is available and meets manufacturer's requirements.

Preparation

Prepare opening to permit correct installation of door unit to perimeter air and vapor barrier seal.

Apply primer to wood frame.

Installation

Install door unit assembly in accordance with manufacturer's instructions.

Anchor assembly to wall construction and building framing without distortion or stress.

Securely brace door tracks suspended from structure. Secure tracks to structural members only.

Fit and align door assembly including hardware.

Coordinate installation of electrical service. Complete power and control wiring from disconnect to unit components.

Install << perimeter trim;  closures; and  ___________>>.

Tolerances

Maximum Variation from Plumb:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Maximum Variation from Level:  << 1/16 inch (1.5 mm); or  ___ inch (___ mm)>>.

Longitudinal or Diagonal Warp:  Plus or minus << 1/8 inch in 10 feet (3 mm in 3 m); or  _____ inch in 10 feet (_____ mm in 3 m)>> straight edge.

Maintain dimensional tolerances and alignment with adjacent work.

Adjusting

Adjust door assembly for smooth operation and full contact with weatherstripping.

Confirm operation is in accordance with manufacturer's instructions with present field representative. Identify adjustments to door assembly.

Cleaning

See Section << 017000 - Execution and Closeout Requirements>> for additional requirements.

Clean doors and frames<<  and glazing;    _____; or  None - N/A>>.

Remove<<  temporary;    _____; or  None - N/A>> labels and visible markings.

Protection

Protect installed products from damage until Date of Substantial Completion.

Do not permit construction traffic through overhead door openings after adjustment and cleaning.

END OF SECTION